| Employability Skills & Mini Project |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class & Batch |  | : | TE-7, L-7 |  | Group No. |  | : | L7_3 |
| Synopsis on |  |  |  |  |  |  |  |  |
| “Power Saving Using APFC Unit” |  |  |  |  |  |  |  |  |
| Submitted By |  |  |  |  |  |  |  |  |
| Roll No. | Name of the Student |  |  | e-mail id |  | Mobile No. |  |  |
| 3726 | Shubham S. Kale |  |  | shubhamkale555.sk@gmail.com |  | 7028488287 |  |  |
| 3728 | Rucha N. Khopkar |  |  | ruchakhopkar@gmail.com |  | 7262035298 |  |  |
| 3735 | Kshitij S. Mahajan |  |  | kshitijmahajangh@gmail.com |  | 8796416746 |  |  |
| Project Guide Prof.V.B. Vaijapurkar |  |  |  |  |  |  |  |  |
| DEPARTMENT OF ELECTRONICS AND TELECOMMUNICATIONS PUNE INSTITUTE OF COMPUTER TECHNOLOGY |  |  |  |  |  |  |  |  |
| Academic Year : 2017 - 2018 |  |  |  |  |  |  |  |  |

INTRODUCTION :

Majority of the loads in the industries are highly inductive in nature such as induction motors, AC/DC drives, welding machines, arc furnaces ,  fluorescent Lightings, electronic controls and computers. There may be a few resistive loads for heaters and incandescent bulbs. Very rarely industries may  have   capacitive loads such as synchronous motors [1].

Net industrial load is highly inductive causing a very poor lagging   power factor. If this poor power factor is uncorrected,the industry will require  a   high maximum demand from Electricity Board and also will suffer a penalty for poor power factor.  Standard   practice is to   connect power capacitors in the power system at appropriate places to compensate the inductive nature of the load.

The automated power factor corrector (APF) using capacitive load bank is helpful in providing the power factor correction. Proposed   automated  project involves measuring the power factor value from the load using microcontroller. The design of  auto-adjustable power factor correction is to ensure the entire power system always preserving unity power factor.

LITERATURE SURVEY :

In earlier days , the power factor correction was done by using the resistive loads which provides the more power loss at the output. In second method , the power factor correction was done by using the capacitive load bank   which was done by manual changes but it gives only less amount of increasing in efficiency.

There    are    also some   other   complicated   methods    invented   for PF correction   which  are not much popular for    economical    purpose and some methods  are  under  research .Phase  Advancer ,Three -Phase buck -boost PFC circuit and controlling method etc.are some other under research methods.

One more method is  Synchronous Condenser .It is synchronous motor that rotaes under no load condition.Asynchronous motor shows capacitive behaviour while operating in over-excited mode .By controlling the field excitation power factor can be adjusted continuously .It provides step-less PF correction and not affected by system harmonics.But its installation and maintenance is costly[2].

PROBLEM DEFINITION AND OBJECTIVES:

An electrical load that operates on alternating current requires apparent power, which consists of real power plus reactive power. Real power is the power actually consumed by the load. Reactive power is      demanded   by the load and returned to the power source, and it   is the cyclical      effect     that occurs when alternating current passes through a load that contains a reactive component.

The presence of reactive power causes the real power to be less than the apparent power, and so, the electric load has a power factor of less than 1.  The reactive power increases the current flowing between the power source and the load, which increases the power losses through     transmission and   distribution lines. This results in operational and financial losses for power companies.

Therefore, power companies require their customers, especially those with large loads, to maintain their power factors above a specified amount. The simple correction techniques described above do not cancel out the reactive power at harmonic frequencies, so more sophisticated techniques must be used to correct for non-linear loads [3].

Objectives:1. Eliminating the power factor penalty.

2. Increased system capacity and reduced system losses in your

electrical system.

3. Increased voltage level in your electrical system and cooler, more

efficient motors.

4. Reducing peak KW billing demand.

BLOCK DIAGRAM:

Fig.1 Block Diagram.

1.Potential Transformer: Transformers convert AC electricity from one voltage to another with a little loss of power. The potential transformer (PT) essentially uses a step-down transformer to reduce the dangerously high voltage to a safer low voltage (typically 110 Volts) in any substation.

2.Current Transformer: In an electrical circuit, a current transformer (CT) is used for measurement of electric currents. When current in a circuit is too high to directly apply to measuring instruments, a current transformer produces a reduced current accurately proportional to the current in the circuit, which can be conveniently connected to measuring and recording instruments.

3.Zero Crossing Detector: The zero crossing detector is a sine-wave to square-wave converter. To calculate the phase lag between the voltage and current, two detectors are used to find the arrival instance of each signal.

4.Display Unit: One of the most common devices attached to the microcontroller is an LCD display. Some of the most common LCDs connected to microcontroller are 16x2 and 20x4 displays.

5.Microcontroller: It takes input, processes it according to the program written to it and then gives the processed output. It also has some on chip memory which is utilized to store some temporary variables during processing is going on.

6.Relay unit: This unit consists of a relay driver and some relays. As the output of the microcontroller cannot control the switching of capacitors directly, this unit is responsible for controlling the high power circuit from a low power circuit.

7.Capacitor Bank: Shunt capacitor banks are used to improve the quality of the electrical supply and the efficient operation of the power system. Studies show that a flat voltage profile on the system can significantly reduce line losses.

HARDWARE REQUIREMENTS:

| Sr NO. | Component Name |
| --- | --- |
| 1 | LCD Display |
| 2 | PIC Microcontroller |
| 3 | ULN2003 Relay Driver |
| 4 | Relay |
| 5 | Capacitor Bank |
| 6 | Voltage Transformer |
| 7 | Current Transformer |
| 8 | Zero Crossing Detector(LM358) |

SOFTWARE REQUIREMENTS:

| SR No. | Software Name |
| --- | --- |
| 1. | Microchip MPlab |
| 2. | Eagle |
| 3. | Proteus |
| 4. | Multisim |

REFERENCES:

1.en.wikipedia.org/wiki/Power_factor_correction.

2.”Energy Saver power Factor Controller For Synchronous Motors”.IEEE

Transactions on Power Apparatus And Systems.

3.Jos Arrillaga,”Power System Harmonics “.

4.”Power Factor Technical “ available at www.researchgate.com.

5.Minimizing Penalty In Industrial Power Consumption By IRJET.

https://www.irjet.net/archives/V4/i3/IRJET-V4I3366.pdf.

But its installation and maintenance is costly [11].

B. Static Capacitor Bank

Capacitors causes leading power factor as it shifts current

ahead of the voltage. So to correct lagging power factor, it is a

convenient method for which this method is practiced

worldwide vastly. Though it has some limitations like the

inability to absorb harmonics and doesn’t provide step-less

correction, it is a popular choice for PFC for its low cost of

installation and maintenance [11].

C. Others Methods

There are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods

step-less PF correction and not affected by system harmonics.

But its installation and maintenance is costly [11].

B. Static Capacitor Bank

Capacitors causes leading power factor as it shifts current

ahead of the voltage. So to correct lagging power factor, it is a

convenient method for which this method is practiced

worldwide vastly. Though it has some limitations like the

inability to absorb harmonics and doesn’t provide step-less

correction, it is a popular choice for PFC for its low cost of

installation and maintenance [11].

C. Others Methods

There are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods

step-less PF correction and not affected by system harmonics.

But its installation and maintenance is costly [11].

B. Static Capacitor Bank

Capacitors causes leading power factor as it shifts current

ahead of the voltage. So to correct lagging power factor, it is a

convenient method for which this method is practiced

worldwide vastly. Though it has some limitations like the

inability to absorb harmonics and doesn’t provide step-less

correction, it is a popular choice for PFC for its low cost of

installation and maintenance [11].

C. Others Methods

There are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods

step-less PF correction and not affected by system harmonics.

But its installation and maintenance is costly [11].

B. Static Capacitor Bank

Capacitors causes leading power factor as it shifts current

ahead of the voltage. So to correct lagging power factor, it is a

convenient method for which this method is practiced

worldwide vastly. Though it has some limitations like the

inability to absorb harmonics and doesn’t provide step-less

correction, it is a popular choice for PFC for its low cost of

installation and maintenance [11].

C. Others Methods

There are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods

step-less PF correction and not affected by system harmonics.

But its installation and maintenance is costly [11].

B. Static Capacitor Bank

Capacitors causes leading power factor as it shifts current

ahead of the voltage. So to correct lagging power factor, it is a

convenient method for which this method is practiced

worldwide vastly. Though it has some limitations like the

inability to absorb harmonics and doesn’t provide step-less

correction, it is a popular choice for PFC for its low cost of

installation and maintenance [11].

C. Others Methods

There are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods.

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods.

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods.

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods.

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods.

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods.

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.

are some other under research methods.

here are also some other complicated methods invented for

PF correction which are not much popular for economical

purpose and some methods are under research. Phase Advancer,

Three-phase buck-boost PFC circuit and controlling method etc.